Construction Inspection Checklist

Concrete Pavement Inspection

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Materials |
|  | 1. Does the mix design conform to Class P? |
|  | 2. Are the dowels straight, smooth, coated and capped to allow free movement? |
|  | 3. Are the dowels secured (no movement) and within ¼” tolerance? |
|  | 4. Are Grade 40 rebars @ 2/3 spacing used where bending is required? |
|  | 5. Are the tie bars type single or multi-piece? (required to develop a tensile strength over their entire length equal to 1-1/4 times the yield strength of the tie bars shown) |
|  | Mixer |
|  | 6. What mixer type is used? (Transit mix allowed only by plan note) |
|  | Forms / Slip-form |
|  | 7. Is form or slip-form placement method used? |
|  | 8. Are the forms secured to prevent movement? |
|  | 9. Is the form depth within 2” of the pavement depth? |
|  | 10. If forms of a depth less than the pavement edge are used, are they brought to the required edge thickness by securely attaching metal strips or wood shims to the full width and length of the base of the forms, except as allowed in Article 360.6(2)? |
|  | 11. If the slip-form placement method is used, is the slip-form track stable, smooth, and to grade? |
|  | Placing and Consolidation |
|  | 12. Is mechanical spreading used except as allowed by the plans? |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | 13. Has the frequencies of the vibrators been checked and do they meet the specification requirements? |
|  | 14. Is the spacing of internal vibrators within 24”? |
|  | 15. Are all vibrators working and not leaking? |
|  | 16. Does the time between mixing and placement meet specification requirements for the temperature, use of retarder, and haul unit? |
|  | 17. Is the concrete placed so that rehandling and segregation are minimized? |
|  | 18. When concrete is placed, is the air temperature at least 35 degrees F and rising? |
|  | 19. If concrete is placed directly on subgrade or unstabilized based, is the subgrade or unstabilized based stable and moistened before concrete is placed? |
|  | Finishing and Curing |
|  | 20. Is mechanical finishing performed except for short irregular sections or otherwise permitted in plans? |
|  | 21. Does the finished surface meet the specification requirements when checked with a 16 ft straightedge? |
|  | 22. Does the surface have a coarse texture satisfactory to the Engineer? |
|  | 23. Is timely and adequate curing of the concrete performed? |

Comments: