Construction Inspection Checklist

Reinforcing Steel

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Order Lists |
|  | 1. On the reinforcing steel order lists, were the same respective bar marks used for labeling as shown on the plans? |
|  | 2. Were the vertical reinforcement in columns, walls, piers, and shafts not ordered until elevations were established in the field? |
|  | Identification |
|  | 3. Was the bar reinforcement shipped in standard bundles, tagged and marked according to CRSI Manual of Standard Practice? |
|  | Bending |
|  | 4. Were reinforcing bars fabricated according to ACI SP 66? |
|  | 5. Were all reinforcing bars that required bending, cold bent? |
|  | 6. Were the overall height or drop bend tolerances of deck truss bars to plan requirements? |
|  | 7. If bars partially embedded in concrete were bent were they bent as shown on the plans or otherwise permitted? |
|  | 8. If the dimensions of hooks or the diameter of bends were not shown on the plans were standard hooks conforming to ACI SP 66 supplied? |
|  | Protection of Material |
|  | 9. Was the reinforcing steel stored above the ground on platforms, skids or other supports? |
|  | 10. Was reinforcing steel protected from physical damage, rust and other surface deterioration? |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | 11. Was reinforcing steel used only when the surface was clean and the minimum dimensions, cross-sectional area, and tensile properties conformed to the physical requirements for the size and grade of steel specified? |
|  | 12. Was reinforcing steel that was cracked, laminated, or was covered with dirt, rust, loose scale, paint, grease, oil, or other deleterious material not allowed to be used? |
|  | Epoxy Coated Reinforcing Steel |
|  | 13. Were coated bars supported on padded contact areas and all bundle bands padded? |
|  | 14. Was bar-to-bar abrasion prevented? |
|  | 15. Were bundles of epoxy coated reinforcing steel not allowed to be dropped or dragged? |
|  | 16. Before placement were coated bars inspected for damage to the coating? |
|  | 17. Were all defects patched in the coating that were discernable to the unaided eye with a prequalified patching/repair material according to AASHTO M 284? |
|  | 18. Were all areas needing patching cleaned thoroughly and all rust removed before patch was applied? |
|  | 19. Were all patch applications done promptly after cleaning according to the resin manufacturer’s recommendations and before detrimental oxidation occurred? |
|  | 20. Was the patching material overlapped onto the original coating for 2 inches or as recommended by the manufacturer? |
|  | 21. Was a minimum 8-mil dry film thickness provided on the patched areas? |
|  | 22. Was any damage to the coating, noted after installation, cleaned and patched? |
|  | 23. Were field repairs not allowed to be performed on bars that had severely-damaged coatings? |
|  | 24. Were bars with severely damaged coatings replaced? (Severely-damaged coating is defined as a coating with a total damaged area in any 1 foot length of bar that exceeds 5 percent of the surface area of that portion of the bar) |
|  | 25. Were mechanical splices coated after splice installation, according to AASHTO M 284 for patching damaged epoxy coatings? |
|  | Placing and Fastening |
|  | 26 Were the bars supported, on precast concrete blocks or metal supports according to CRSI Manual of Standard Practice? |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | 27. Were concrete block supports attached to the support bar with wire cast in the center of each block? |
|  | 28. Were class 1 (plastic protected) or class 2, type B (stainless steel protected) metal supports used in contact with exposed concrete surfaces? |
|  | 29. Was stainless steel used that conformed to ASTM A 493, type 430? |
|  | 30. Were chairs, tie wires, and other devices used to support, position or fasten epoxy-coated reinforcement, coated with a dielectric material? |
|  | 31. Were plastic supports not allowed to be used? |
|  | 32. Were slab bars spaced no more than 4 feet apart transversely or longitudinally? |
|  | 33. Were bar supports not allowed to be used either directly or indirectly to support runways for concrete buggies or other similar construction loads? |
|  | 34 Were parallel bars spaced within 1-½ inches of the required location? |
|  | 35. Were spacing variations not cumulated? |
|  | 36. Did the average of any two adjacent spaces not exceed the required spacing? |
|  | 37. Was a 2-inch clear cover provided for all reinforcement except as otherwise shown on the plans? |
|  | 38. Was reinforcing steel placed in deck slab within ¼ inch of the vertical plan location? |
|  | 39. Were bridge deck reinforcing bars tied together at all intersections except where spacing was less than 12 inches in both directions, in which case alternate intersections were tied? |
|  | 40. Was the clear cover over deck reinforcing steel checked using a template before placing deck concrete? |
|  | 41. Were damaged supports replaced? |
|  | 42. Were bundle bars tied together at intervals not exceeding 6 feet? |
|  | 43. Were bars bundled and if so was the location and splice details specified? |
|  | 44. Was concrete not allowed to be placed in any member before reinforcement was approved? |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Splices |
|  | 45. Was splicing used and if so was it shown on the plans and if not shown on the plans was approval given? |
|  | 46. Was splicing of reinforcing bars only used where shown on the plans or accepted drawings and if so were the proper overlap lengths used? |
|  | 47. Were slab bar mechanical splices not placed adjacent to each other? |
|  | 48. Were lapped splices made by placing the reinforcing bars in contact and wiring them together so as to maintain the alignment and position of the bars? |
|  | 49. If welding of reinforcing steel was permitted by the contract, did the welds conform to AWS D 1.4? |
|  | 50. Were the welders used, currently certified? |
|  | 51. If the contract required, was each weld tested using magnetic particle, radiography, or other nondestructive inspection techniques? |
|  | 52. If mechanical couplers were approved for use, did the couplers have a strength that was at least 125 percent of the required yield strength of the reinforcing steel? |
|  | 53. If welded wire fabric was shipped in rolls, were fabric rolls straightened into flat sheets before placing? |
|  | 54. Were sheets of mesh or bar mat reinforcement spliced by overlapping not less than 1 mesh width plus 2 inches and securely fastened at the ends and edges? |

Comments: