Construction Inspection Checklist

Manholes, Inlets, and Catch Basins

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | General |
|  | 1. Was excavation and backfill completed according to plans? |
|  | Concrete Construction |
|  | 2. Were concrete manholes, inlets, and catch basins completed according to plans? |
|  | 3. Was the surface finished according to plans? |
|  | 4. Where a pipe enters through an existing concrete wall, was the concrete and steel reinforcement cut in a manner that did not loosen the reinforcement in the wall? |
|  | 5. Was the steel reinforcement cut flush with the opening wall face? |
|  | 6. Were all joints and openings that were cut in the wall grouted? |
|  | 7. Were the channel flow lines in manholes, inlets, and catch basins finished accurately to match the pipe flow line? |
|  | 8. Were precast concrete manhole sections assembled with flexible watertight gaskets or mastic joint fillers in the tongue and groove joints? |
|  | 9. If gaskets were used, were the precast units handled carefully after the gasket had been attached to avoid damaging the gasket or contaminating the joint? |
|  | 10. Was the proper alignment attained before the joints were forced home? |
|  | 11. Was partial support maintained during the insertion of the tongue or spigot to minimize the unequal lateral pressure on the gasket and to maintain concentricity until the gasket was properly positioned? |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | 12. If mastic was used, was the mastic joint filler applied according to the manufacturer’s recommendations? |
|  | 13. Were metal frames set in a full joint mortar bed? |
|  | 14. Were all joints and openings made watertight by using grout or a preformed joint sealer? |
|  | 15. Were mortar joints finished with a bead on the outside and a smooth finish on the inside? |
|  | 16. Were ladder rungs spaced uniformly on 12-inch centers and aligned vertically? |
|  | 17 Were ladder rungs grouted into precast concrete walls? |
|  | Masonry Block Construction |
|  | 18. Were the concrete footings constructed according to plans? |
|  | 19. Was the block masonry constructed plumb? |
|  | 20. Were the vertical joints staggered and the block cells set vertical? |
|  | 21. Was the block dampened to reduce absorption? |
|  | 22 Were the bearing members and vertical joints buttered full of mortar? |
|  | 23. Were all sides of block bonded with mortar? |
|  | 24. Were all joints constructed straight, level, plumb, flush, and ¼ to ½ inch thick? |
|  | 25. Was the masonry block structure allowed to cure according to plans? |
|  | Metal Construction |
|  | 26. Were the metal drop inlets fabricated from the same material as the adjoining metal pipes? |
|  | Grade Adjustment of Existing Structures |
|  | 27. Were metal frames and grates adjusted to grade before placing the surface course? |
|  | 28. Were frames, covers and grates removed and cleaned? |
|  | 29. Were the walls trimmed down to solid material? |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | 30. Were the walls reconstructed with the same material as existing and were the cleaned frames reset at the required elevation? |
|  | 31. When catch basins and inlets were adjusted to grade and abutting existing concrete construction, were the castings separated from the adjacent concrete with a preformed expansion joint no less than ½ inch thick? |
|  | 32. Was each structure cleaned of all accumulated silt, debris, or foreign matter? |
|  | 33. If an existing structure was abandoned were all pipes entering the structure sealed with a tight fitting plug of concrete not less than 6 inches thick or water tight masonry not less than 8 inches thick? |
|  | 34. Was the base of the concrete structure fractured to prevent entrapment of water? |
|  | 35. Was the top of the structure obliterated to an elevation at least 3 feet below finished grade and backfilled according to plans? |

Comments: