Construction Inspection Checklist

Waterproofing

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Membrane Waterproofing |
|  | 1. Was Waterproofing Membrane installed according to plans? |
|  | Membrane Waterproofing with Mortar Protection |
|  | 2. Was the membrane completely covered, except on under cut surfaces, with a course of reinforced mortar?) |
|  | 3. Did the mortar course use welded wire reinforcement (W5.5 by W5.5) or its equivalent midway between the top and bottom surfaces of the mortar? |
|  | 4. Was the top surface of the mortar course troweled to a smooth, hard finish? |
|  | 5. Was the protective mortar cured with wet burlap held in close surface contact for 72 hours? |
|  | 6. Was any undercut membrane surfaces protected with a layer of asphalt roofing laid in hot asphalt and was the same bituminous material used for mopping the membrane? |
|  | Membrane Waterproofing with Asphalt Plank Protection |
|  | 7. If the membrane was covered with asphalt plank, was the plank laid in regular, straight courses? |
|  | 8. Were whole planks used in all cases except as required for closures and for fitting around openings and obstructions? |
|  | 9. Was all surplus talc or other powder removed from the planks with a stiff brush or broom before being laid? |
|  | 10. Was each piece laid in a mopping of hot asphalt, and were the edges and ends of the pieces coated with hot asphalt before placing an adjacent piece in contact? |

|  | 11. Was each individual piece tightly pressed against the piece next to it? |
| --- | --- |
|  | 12. Was the completed surface uniform and smooth without open joints? |

Comments: